# AI Execution Guide — Space-Based Industrial Fire Classification

Internal Team Reference | AI Input / Output / Decision Layer Only (Frontend, backend, and GIS visualization should be  covered in separate team documents and are out of scope here.)

## 1. Problem Statement

Satellite-based thermal monitoring systems (e.g., NASA FIRMS) can detect that a thermal anomaly is occurring, but not what it is. A forest fire, a controlled stubble burn, an industrial gas flare, and a refinery explosion are all currently indistinguishable on raw hotspot feeds. This system closes that gap: it ingests thermal, spatial, environmental, and historical data, and outputs a classified, ranked hazard with an associated confidence and recommended action.

## 2. What the AI System Must Do

Given a real-time thermal detection, the system must:

- Enrich the raw hotspot with spatial, historical, and meteorological context.

- Classify it into one of five hazard tiers (Section 3).

- For high-severity classes, delineate the precise spatial extent of the fire/plume.

- Output a structured, machine-readable result that downstream systems (alerting, GIS, ops dashboards — handled by other team workstreams) can consume.

## 3. Classification Taxonomy

| Class | Type | Urgency Tier | Primary Risk |
| --- | --- | --- | --- |
| 1 | Accidental Industrial Fire / Explosion | Critical Emergency | Infrastructure destruction, toxic leaks, mass casualties, domino-effect explosions |
| 2 | Wildfire / Uncontrolled Forest Fire | High Emergency | Ecological destruction, rapid spread, threat to settlements |
| 3 | Uncontrolled Mining / Coal Seam Fire | Moderate Emergency | Ground subsidence, toxic subterranean gas, pit instability |
| 4 | Agricultural / Stubble Burning | Low / Seasonal | Air quality degradation, localized soil impact, compliance |
| 5 | Persistent Flare / Routine Industrial Source | Routine / Operational | Standard operational heat within permissible limits |

Classes 1 and 2 trigger immediate downstream escalation (see Section 9, Decision Logic).

## 4. AI Task Allocation: Deterministic Pipeline vs. AI Inference

To keep latency low and reliability high, physics/geometry problems are not handed to a model. Only genuine pattern-recognition problems go to AI.

Deterministic (non-AI) pipeline:

- Point-in-Polygon spatial verification (PostGIS / H3) against industrial boundaries

- Slope gradient (θ) from Copernicus 30m DEM

- 90-day persistence ratio, historical FRP mean/std, and Z-score computation

- Dual-band Planck blackbody inversion for sub-pixel combustion temperature

- Plume advection via vector addition of real-time GFS wind vectors (see Section 10 — this is a first-order simplification; flag as a known limitation, not a bug)

AI inference pipeline:

- Multi-modal triage classification: mapping the 21-feature tabular vector to one of 5 hazard classes

- Multi-spectral spatial segmentation: fire perimeter and smoke plume delineation from imagery

## 5. Model Architecture

### Stage 1 — Primary Triage Classifier

- Model: CatBoost (Categorical Boosting)

- Task: Continuous real-time classification of every incoming thermal trigger into Classes 1–5

- Input: 21-dimensional tabular vector (mixed numeric + categorical) — full spec in Section 6

- Why CatBoost: Native handling of high-cardinality categoricals (land-cover IDs, OSM tags) without one-hot blowup; supports custom cost matrices to penalize Class 1 false negatives; TreeSHAP explainability for post-hoc audit of any alert

- Rejected alternatives: TabNet (overfitting risk, opacity, not worth it at this data scale); Random Forest (struggles with the ~10,000:1 class imbalance and gives poorly calibrated probability outputs)

- Source: Yandex Open-Source (catboost/catboost)

### Stage 2 — Tactical Vision Segmenter

- Model: SegFormer (MiT-B0 backbone) or lightweight U-Net

- Task: Triggered only on Class 1/2 alerts — maps exact burn/smoke pixel boundaries at 20m resolution

- Input: Sentinel-2 multi-spectral cutout (SWIR 12, SWIR 11, NIR 8, Red 4) as an H×W×C tensor

- Why segmentation over detection/classification: YOLO-style bounding boxes swallow large non-burning background and can't trace irregular fire lines; ResNet/ViT-style whole-tile classification can't localize which specific asset within an industrial complex is burning. A pixel mask is what tactical response actually needs.

- Rejected alternatives: YOLO/RT-DETR, ResNet/ViT classification

- Source: Hugging Face Transformers (SegFormer) or SMP (qubvel/segmentation_models.pytorch)

Why two stages, not one: Running Stage 2 continuously across the whole subcontinent is computationally infeasible in real time. Stage 1 is the cheap, always-on "tripwire"; it only calls Stage 2 when the stakes justify the compute.

## 6. Feature Matrix — Full 21-Feature Specification

This is the exact input vector Stage 1 (CatBoost) consumes. Every row should map directly to a column in the training DataFrame.

### A. Radiometric Features (NASA FIRMS Feed)

| # | Feature | Description |
| --- | --- | --- |
| 1 | bright_ti4 | Brightness temperature, VIIRS 3.75 µm channel (K) |
| 2 | bright_ti5 | Brightness temperature, VIIRS 11.0 µm channel (K) |
| 3 | temp_ratio | bright_ti4 / bright_ti5 — proxy for sub-pixel combustion intensity |
| 4 | frp | Fire Radiative Power (MW) — instantaneous thermal energy output |
| 5 | scan, track | Pixel footprint deformation at off-nadir swath boundaries |
| 6 | daynight | Binary D/N flag — industrial sources burn 24/7; stubble burning drops to ~zero at night |

### B. Spatial & Topological Features (OSM & ESA WorldCover)

| # | Feature | Description |
| --- | --- | --- |
| 7 | is_in_industrial_polygon | Point-in-polygon result against OSM industrial vector layers |
| 8 | distance_to_industrial_m | Euclidean distance to nearest industrial parcel (m) |
| 9 | facility_type | Categorical: refinery, chemical, steel_mill, power_plant, none |
| 10 | lulc_class | ESA WorldCover 10m class at hotspot center (Tree/Shrub/Crop/Built-up) |
| 11 | lulc_entropy_500m | Shannon entropy of land-use classes within 500m — separates urban from forest context |

### C. Spatio-Temporal Baseline & Kinematics (90-Day Buffer)

| # | Feature | Description |
| --- | --- | --- |
| 12 | persistence_score | P = detections / total overpasses, trailing 90 days |
| 13 | baseline_frp_mean | Historical mean thermal power at this coordinate |
| 14 | baseline_frp_std | Historical FRP variance at this coordinate |
| 15 | frp_z_score | Z = (FRP_current − baseline_mean) / baseline_std |
| 16 | centroid_drift_velocity | Fire-front movement (m/hr) vs. previous pass |
| 17 | cluster_pixel_count | Connected active-fire pixels within 1km |

### D. Multi-Spectral Remote Sensing Features (Sentinel-2 MSI)

| # | Feature | Description |
| --- | --- | --- |
| 18 | swir_nir_ratio | Band 12 (2.19 µm) / Band 8 (0.84 µm) — isolates ultra-high-temp flaring |
| 19 | delta_nbr | Pre- vs. post-fire Normalized Burn Ratio change — canopy damage |
| 20 | delta_ndvi | NDVI crash — vegetation loss magnitude |
| 21 | thermal_plume_spread | Bounding box area of high-SWIR anomaly (m²) |

Team note: Features 1–6 and 12–17 are available at trigger time from FIRMS + the internal SQL baseline store (cheap, always computed). Features 7–11 require a PostGIS/vector lookup (cheap, always computed). Features 18–21 require an on-demand Sentinel-2 pull and are only needed for Stage 2, not Stage 1 — don't block the Stage 1 tripwire on a Sentinel-2 fetch.

## 7. Live Data Ingestion Strategy

Model 1 (tripwire) feeds:

- Thermal trigger — polled every 15–30 min from NASA FIRMS NRT REST API (VIIRS & MODIS)

- Contextual enrichment (triggered instantly on each new hotspot):

  - Weather: Open-Meteo API (GFS)

  - Spatial/terrain: local PostGIS store (OSM vectors) + OpenTopography API (Copernicus DEM)

  - Historical baseline: internal SQL table tracking 90-day regional FIRMS detections

Model 2 (vision) feeds:

- Queried on-demand from Copernicus Data Space Ecosystem (CDSE) OData/STAC API only upon a Class 1 or 2 trigger — pulls the most recent Sentinel-2 L2A tile intersecting the hazard coordinates.

## 8. Training & Data Preparation

### 8.1 The Core Challenge: No Pre-Labeled Dataset

There is no existing dataset mapping the 21 features to our 5-tier taxonomy. FIRMS gives raw coordinates, not labels. We solve this with distant supervision — programmatic labeling by crossing historical thermal points against geospatial boundary rules — using the NASA FIRMS Historical Archive (VIIRS 375m, India, past 2–3 years) processed with GeoPandas.

### 8.2 Automated Labeling Rules

- Class 5 (Flares/Smelters): Coordinate (rounded to 3 decimals) registers a thermal anomaly on >70% of days in the year → auto-label Class 5. Cross-verify against NOAA VIIRS Nightfire.

- Class 4 (Stubble): Coordinate intersects an ESA WorldCover Cropland polygon (Class 40) and falls in a peak harvest window (e.g., Oct–Nov in Punjab/Haryana) → auto-label Class 4.

- Class 3 (Mining): Coordinate intersects an OSM landuse=quarry or mine polygon → auto-label Class 3.

- Class 2 (Wildfire): Coordinate intersects an ESA WorldCover Tree Cover polygon (Class 10) and shows high FRP → auto-label Class 2.

- Class 1 (Industrial Disasters): Too rare to auto-label. Manually curate 10–20 major historical Indian industrial fires (e.g., Baghjan blowout, Vizag refinery fire), locate coordinates/dates in the FIRMS archive, and hand-label those rows.

Refinement — Class 3 vs. Class 5 tie-break rule: A persistent coal-seam fire can satisfy the Class 5 ">70% of year" persistence rule just as easily as a genuine flare stack. Apply rule precedence: OSM mining-polygon intersection (Class 3) overrides persistence-based Class 5 labeling. Check is_in_industrial_polygon / facility_type mining tags before evaluating the persistence threshold, not after.

### 8.3 ⚠️ Known Risk — Label Circularity (must be addressed before training)

The labeling rules above use persistence_score, frp_z_score, and lulc_class to generate labels — but those same fields are also input features (7, 10, 12, 15 in Section 6). Left unaddressed, CatBoost will partly learn to reverse-engineer our own labeling heuristic rather than the underlying phenomenon, and reported validation accuracy will be optimistic.

Required mitigation:

- Carve out a held-out, human-verified test set (target: 300–500 rows, stratified across all 5 classes) that is manually checked against satellite imagery / news reports / OSM — not generated purely by the auto-label rules.

- This set is used only for final evaluation, never for training. It's the number you actually trust and report internally.

- If Stage-1 performance drops materially on this set vs. the auto-labeled validation split, that gap is the circularity effect — treat it as the real error rate.

### 8.4 Feature Matrix Construction

- Spatial lookups append slope (θ) and distance to nearest industrial polygon from local raster/vector files.

- Temporal baselines: script groups historical FIRMS data to compute 90-day FRP mean, std, and Z-score per row.

- Final output: a Pandas DataFrame with 21 feature columns (numeric + categorical) + 1 label column (1–5).

### 8.5 Model 1 (CatBoost) Training Protocol

- Loss: Native MultiClass loss, 5 outputs.

- Class imbalance handling: auto_class_weights='Balanced' — reweights the loss gradient so the model is heavily penalized for missing a Class 1 event, instead of lazily predicting Class 5 for everything.

- Categorical handling: categorical feature indices (facility_type, lulc_class, etc.) passed directly via cat_features, no one-hot encoding needed.

- Hardware: CPU + system RAM is sufficient; 1–2M rows trains in minutes on a standard laptop, seconds with GPU.

Refinement — Class 1 data scarcity mitigation: 10–20 manually curated Class 1 examples is not enough for CatBoost to learn a robust boundary on its own, no matter how the loss is weighted — class weighting fixes gradient emphasis, not information scarcity. Run an anomaly-detection model in parallel (Isolation Forest or One-Class SVM) on the deterministic features most diagnostic of an industrial disaster (frp_z_score, persistence_score, swir_nir_ratio, centroid_drift_velocity). Flag as Class 1 candidate if either CatBoost predicts Class 1 or the anomaly detector flags an outlier inside an industrial polygon. This gives a second, independent detection path for the highest-stakes, lowest-data class instead of relying on one small-sample classifier alone.

### 8.6 Model 2 (SegFormer) Training Protocol

- Base weights: ImageNet-pretrained MiT-B0 backbone.

- Transfer learning: Fine-tune first on open-source EO fire-boundary datasets (Kaggle Wildfire, Omdena).

- Custom curation: Sentinel-2 SWIR cutouts of known industrial disasters (from the Class 1 manual list), annotated in CVAT or Roboflow for precise ground-truth masks.

Refinement — quantify the custom annotation target: Don't leave this open-ended. Target 200–500 manually annotated tiles as the custom industrial-plume set, expanded via augmentation (rotation, flip, brightness/contrast jitter) to increase effective training volume without requiring proportionally more manual annotation hours. Track this as a concrete team task with an owner and a deadline, not a background activity.

- Hardware: MiT-B0 is light enough to train locally on a single consumer GPU (e.g., RTX 3060/4060, 6GB+ VRAM) using FP16 mixed precision and small batch sizes.

## 9. Validation and Testing Strategy

### 9.1 Spatial Block Cross-Validation

Random train/test splits fail on geospatial data — the model can simply memorize coordinates of known flares. Instead: divide the subcontinent into a 50km × 50km grid, hold out entire grid cells for validation. This forces the model to classify unseen geography using only the engineered features (slope, Z-score, LULC), not memorized location.

### 9.2 Temporal Holdout

Train on 2023–2024 data, test on 2025–2026 data, to guard against seasonal shift and satellite sensor recalibration drift over time.

### 9.3 Evaluation Metrics

Accuracy is discarded — it's meaningless under this class imbalance. Use instead:

- Macro F1-Score — balanced precision/recall across all 5 classes

- Class 1 Recall (primary metric) — target >95%, accepting more false positives (extra flare alerts) to guarantee we don't miss a real refinery explosion

- Refinement — per-class precision, tracked alongside recall: Class 5 is the overwhelming majority class. Macro F1 alone can mask poor precision on it. Track per-class precision explicitly — low Class 5 precision (i.e., lots of routine flares getting misclassified upward) is what drives operator alert fatigue and erodes trust in the system over time, even if Class 1 recall looks great.

- Report all metrics on both the auto-labeled validation split and the held-out human-verified set (Section 8.3) — the gap between the two is your real circularity-adjusted performance.

## 10. Decision Logic — From Model Output to Action

(This is in scope as "decision-making" per the AI layer's responsibility — it stops at handing off a structured decision object, not at how it's displayed.)

- Class 1 or 2 output → immediate Stage 2 trigger (imagery pull + segmentation), regardless of confidence score, given the cost asymmetry favors over-triggering.

- Confidence thresholding: attach CatBoost's class probability to every output. Recommend a two-tier response:

  - High confidence Class 1/2 → auto-escalate to emergency notification pipeline (owned by ops/backend team)

  - Lower confidence Class 1/2, or an Isolation Forest flag without CatBoost agreement → route to a human verification queue rather than auto-escalating, given false alarms on true emergencies carry real credibility cost

- TreeSHAP output should accompany every Class 1/2 alert handed downstream — whoever reviews the alert (human or automated) should see why the model flagged it (which features drove the decision), not just the class label.

- Known limitation to flag to the team: plume advection is currently simple vector addition of GFS wind, not a proper Gaussian dispersion model. This is an acceptable MVP simplification but should not be presented as physically precise — note it as a v2 item if toxic dispersion trajectories become a load-bearing part of the alert.

## 11. Known Limitations — Carry Forward Into V2 Planning

- Label circularity between auto-labeling rules and model input features (mitigated, not eliminated, by the held-out human-verified set).

- Class 1 training data is extremely thin (10–20 examples) — anomaly-detection ensemble is a mitigation, not a full fix. Actively look for more historical Class 1 incidents to expand this set over time.

- Plume advection model is first-order (vector addition), not a full Gaussian plume dispersion model.

- Class 3/5 boundary depends on OSM mining-tag completeness — gaps in OSM coverage for smaller/unofficial mining sites will cause misclassification toward Class 5.

## 13. Data Sourcing & API Endpoints

A crucial component of this architecture is securing the underlying remote-sensing feeds. This system relies entirely on free, open-access Earth Observation (EO) data.

To prevent bottlenecks, the data sourcing strategy is divided into two distinct modes: Historical/Bulk Download (for generating the training datasets) and Real-Time API Interrogation (for the operational deployment).

### 13.1 Historical Sourcing (For Model Training)

Before the models can be trained, you must construct the historical feature matrix. This requires downloading bulk historical data archives.

- NASA FIRMS Active Fire Data (Historical CSVs):

  - Source: NASA Earthdata / FIRMS Archive

  - Access Method: You will use the FIRMS Data Download portal to request historical VIIRS (375m) and MODIS (1km) data.

  - Format: CSV files.

  - Required Action: Download the specific bounding box for the Indian subcontinent covering the past 2–3 years. This will serve as the base dataset for your distant supervision labeling script.

- Copernicus Sentinel-2 Imagery (For SegFormer Training):

  - Source: Copernicus Data Space Ecosystem (CDSE)

  - Access Method: You will query the CDSE catalogue using their STAC (SpatioTemporal Asset Catalog) API or OData REST protocols to locate specific historical tiles.
Remote Sensing Datasets & Sensors Comparison

  - Format: Cloud-Optimized GeoTIFFs (Bands 12, 11, 8, 4).

  - Required Action: Use the manual Class 1 curated list to query the STAC API for historical imagery of those specific disaster dates.

### 13.2 Real-Time API Endpoints (For Operational Deployment)

Once the models are trained and deployed, the backend must continuously poll live feeds to generate the 21-feature input array.

- The Tripwire Trigger (NASA FIRMS NRT API):

  - Endpoint / Source: [https://firms.modaps.eosdis.nasa.gov/api/area/csv/](https://firms.modaps.eosdis.nasa.gov/api/area/csv/)

  - Description: This REST API provides Near Real-Time (NRT) active fire detections.

  - Usage: Your backend will query this endpoint every 15–30 minutes, fetching the latest CSV output for your designated bounding box to trigger the Stage 1 CatBoost triage.

- Tactical Vision Pipeline (Copernicus CDSE STAC API):

  - Endpoint / Source: Copernicus Data Space Ecosystem (CDSE) STAC API ([https://browser.stac.dataspace.copernicus.eu/](https://browser.stac.dataspace.copernicus.eu/))

  - Description: The STAC API is a standardized interface designed by the Earth Observation community to streamline data discovery and retrieval.

  - Usage: When Stage 1 flags a Class 1 or 2 event, the backend will dynamically query this STAC API to find and download the most recent Sentinel-2 Level-2A surface reflectance product intersecting the hazard coordinates.

- Meteorological Advection Data (Open-Meteo):

  - Endpoint / Source: [https://api.open-meteo.com/v1/forecast](https://api.open-meteo.com/v1/forecast)

  - Description: A free, open-source weather API that aggregates NOAA GFS and ECMWF models.

  - Usage: Used to fetch real-time pressure-level wind vectors for the downstream plume advection modeling.